# About the MODES

## Purpose and capability of the program MODES

Program MODES serves for dynamic simulation of electric power system. Its operates as DOS application under WINDOWS (95 and higher).

The MODES makes possible to simulate:

1.	transient stability of generators (loss of synchronism)

2.	middle-term dynamics concerned with the primary control of  frequency (frequency collapse)

3.	long term stability concerned with LFC , OLTC and the excitation limiter  actions (voltage  collapse)

4.	transient phenomena in home consumption of the units (driven start-up, switching over to back-up)

- steady state stability - investigation of the power system for a small deviation  (oscillations)

It can be used for:

a)	Education and study of transients

b)	Studies of large power systems interconnection

c)	Creation of defence against the system failures spreading

d)	Creation of restorations plans after system black outs

e)	Checking and optimisation of the load frequency and system voltage controls

f)	Self-consumption of power plants analysis (motors start-up, back -up and so on)

g)	Harmonic analysis of the power system

The MODES is available in the two form. The first is usual application exactly the package of applications, include the user friendly interface MODMAN (to manage input/output data - so called projects and cases, different auxiliary and additional programs, provide on-line and context sensitive help).

The second form of MODES is a DLL, which is able linking with arbitrary program supporting the DLL in the Microsoft WINDOWS environment. The possibility of the reference to the MODES application is added to the standard MODES models. The necessary interface function for data exchange between applications are developed as well. The DLL form makes possible to implement MODES to the user complex application like energy management system, dispatcher training simulators, security tools and so on.

Fig.  1 The MODES applications overview

The underlined applications are under operation and the others are developed or under consideration.

The Security Assessment identifies transient stability problems due to network contingencies. The Network Simulator  is a decision aided tool for network operators. It can check possible transactions in  the network to prevent thermal overloading, voltage and frequency collapses. The Integrated Network Tool makes possible co-ordination between the electric and thermal networks (power system and district heating systems) by co-generation units regulation.

The aforementioned three applications used real-time data from EMS. The time ranges are several seconds for short term dynamics, minutes for long term dynamics and several hours for operation.

## Workspace

The  workspace is a collection of Projects and other input data for the MODES package applications:

Workspace is physically situated into the MODES working directory (when it has been installed). All information about stored projects is saved in projects archive called PROJECTS.ARC. Project is a collection of cases. All information about stored cases of a project is saved in cases archive called CASES.ARC. Case is a collection of input files for one dynamic calculation. The stored cases are visualized by Projects Tree in the Explorer.

## Data model

The MODES uses three dimensional data model, which is depicted in the following figure.

The input data consist of:

- Load flow data (so called variants)

- Control equipment data

- Dynamic model data (so called modifications)

- Program function data (so called variations).

# About the MODMAN

The interaction between user and program is executed by user interface called MODMAN. It makes possible:

- access to the input and output data (all data is saved in ASCII files)

- projects and cases management

- to run the programs  of package by menu and buttons

- on-line help.

User interface MODMAN is an independent WINDOWS application, which creates the „superstructure“ of the MODES DOS program. The MODES package architecture is depicted in the following figure:

The user interface called MODMAN has the following components:

- Menu

- Toolbar

- Projects explorer

- Load Flow

- Unit Models

- Status bar

The MODMAN provides:

- navigation in workspace by  the Explorer

- projects management by the Project menu

- cases management by the Cases menu

- new projects creation by the Project editor

- new cases creation by the Case editor

- launching the package applications by menu Run or the toolbar

- Context and What this help, Tool Tips and information in the status bar

- access to input files by menu in text or dialogue/ regime and creation of new variations

- load flow data editing by integrated Load Flow Editor and creation of new variants

- dynamic data editing by integrated Unit Model Editor and creation of new modifications.

- access to the user output by the Results menu in text or graphic regime

- access to the reports by the Reports menu

Three stages of help are implemented in the  MODMAN:

- Tool Tips, which is activated by cursor moving above  controls. The Tool Tips can be cancelled by unchecking the check box Tool Tips in the Editors and Dialogues

- What This ? is available in Editors and Dialogues, which have the Question mark button at the upper right corner (there is not Min and Max button). Help is activated by F1 key.

- Context hypertext help is available in the MODMAN and Explorer. Help is activated by F1 key in the context mode or by Help command in the complex mode.

# Power system modelling

The time ranges for the different types of transient phenomena and the equipment, affecting them, are in the figure:

Power system consists of equipment for electricity generation, transmission and distribution included control and information subsystem. The units correspond to the power stations consisting from physical units. The transmission and distribution consists from network and loads. The branches and nodes correspond to these parts of power system in a model. The power system can be divided among areas. The each area can have load frequency control (LFC). The set of the tie lines between two areas is called profile. The abstract entity called automatons, logics and external stabilisers are designed to model of control and protection equipment.

So that the following entities could by modelled in the MODES version 2.2/10:

·	units, which modelled synchronous and asynchronous machines

·	transformers with  on load tape changers

·	branches and nodes with corresponding load

·	node load frequency shedding

·	area load frequency control

·	UPFC

·	automatons, logics, external stabilisers a regulators.

The MODES library contains all usual models for network, generators, prime movers and excitation systems, asynchronous motors, loads and so on. Besides it contains special control models as LFC, four stages load frequency shedding and OLTC.  The conceptual model is in the following figure: