Getting Started

Program MODES serves for dynamic simulation of electric power system. Its operates as DOS application under WINDOWS (95 and higher). The interaction between user and program is executed by user interface called MODMAN. The MODMAN makes possible:

- access to the input and output data (all data is saved in ASCII files)

- projects and cases management

- to run the programs  of package by menu and buttons

- on-line help.

### Basic Terms

- The input and output files are stored in project subdirectories of the working directory. The files contains data for solved problems, which are identified as projects. These projects are shown like tree in the left side of user interface MODMAN. The partial problems called cases are solved in the frame of projects. The case corresponds of a set of input files, which has identity (name and description). The cases may be accessed and recalculated.. The project is a set of cases .with common basic model databases and .parameters catalogues.

- Basic interaction with the MODES is provided by so called graphic. The appearance of graphic may be set by menu „Modify/Graphic“. Time courses of the selected variables may be stored in so called user output files for post processing. The contents of the user output files is set by menu „Modify/ User Files“.

- The network data is the base for dynamic calculation. They are created by nodes (accessed by menu „Modify/ Nodes“) connected by branches (accessed by menu „Modify/Branches“). So called units  (accessed by menu „Modify/Units“) corresponding to the synchronous and asynchronous machines are connected to the nodes.

- The dynamic calculation is a response of dynamic model to the different events, which is modelled system subjected. The MODES software contains library of models for:

- units defined in model database accessed by menu „Modify/Unit Models“

- nodes loads defined in model database accessed by menu „Modify/Nodes Models“

- on load tape changers defined in model database accessed by menu „Modify/Network Models“

- So called database is input file with records linking objects (units. nodes and branches), library models and parameters from so called catalogues. The catalogues store the sets of parameters for dynamic models of:

- generators

- exciters

- excitation control

- additional excitation equipment like power system stabilisers, under/over excitation limiters and secondary Q controller

- turbines

- prime mover control

- additional turbine equipment like speed controller, acceleration relays, by passes and so on

- boilers and PWR reactors

- asynchronous motors

- drived mechanism

- static load

- frequency load shedding

- on load tape changers

- dynamic load

- equivalent (aggregated) asynchronous motor.

So called additional stabilisers may be connected to the unit. The data for additional stabilisers are accessed by menu „Modify/Stabilsers“. The additional stabilisers measure the selected unit or network variable and their output signal is added to excitation of prime mover control of selected unit.  The each so called area may have load frequency control defined by menu „Modify/LFC“. The area is a set of nodes, which have the same area number.

Above all the user can defined so called automaton and logic. The automaton measure the selected unit or network variable and perform predefined action when defined conditions are satisfied. List of possible actions is on the page 3. The logic consist of logical AND and OR elements AND measured logical state of selected automaton (from this point of view is logic superstructure of automatons) and performs the similar actions like automaton. The tables of automatons and logics are accessed by menu „Modify/Automaton" and „Modify/Logics.

The predefined sequence of actions is called scenario. The scenario may be created and modified by menu „Modify/Scenario“.

### Program operation

Package installation

See the CD cover to install the MODES package. Real numbers must be written with decimal point-dot (NO COMMA!) in the input files (or forms and tables). The dot should be set as decimal symbol for editing the input files from the MODMAN in the WINDOWS environment. It may be checked from start-up menu by Settings/Control Panels/Regional Settings/Numbers.. Decimal Symbol.

Run the MODES package

Click on the blue MODES icon in working directory (in explorer or file manager) or workplace to run the user interface MODMAN. You can see the tree of projects on the left. Click double on the projects icon to see embedded cases.. Information about project or case are actualised in the frame Project Specification or Case Specification when you browse Projects tree. Click on the case icon to see set of input files (saved in the project directory) in the Case Files box. Click on the file icon to see the content in the right text box. Use menu „Modify“ to modify this files.

Open the project:

- Click on the project name in the Projects tree.

- Click    icon on toolbar or use menu Project/Open.

- Confirm OK and the MODMAN overwrite working subdirectories  VST a VYST by projects files (use the menu Project/Save or Save As to save proceeding project data).

- Open case:

- Click on the case name in the Projects Cases tree..

- Click  icon se on toolbar or use menu Cases/Open - case icon is getting pink and the MODES is ready recalculate selected case.

#### Run dynamic simulation

- If you have ready project (after load flow calculation and models initialisation) you can click on  or use menu Run/Simulation.

- So called graphic is loaded after input data reading. Graphic shows:

- 1, 2 or 4 charts with up to seven time courses of selected per unit variables in the each chart.

- text box with current simulation time and named variables

- text box with variable identification -symbol, unit, object name and colour

- 9 hotkeys  with pictograms for „on-line“ actions (see the table on the following page)

- two labels in upper part - the right is overridden by program  logging.

- The box End is showed when the final time of the calculation is reached. Press the key E to remove graphic and return to the MODMAN. You can interrupt dynamic simulation by Esc key and continue by C key pressing.

- The number of charts and variable selection is set by menu „Modify/Graphic“.

- The parameters for load flow and dynamic calculation (simulation time, sampling period, integration step range and so on) are set by menu „Modify/Control“.

- Load Flow  calculation

- If you want recalculate the initial load flow you can click on  icon or use menu „Run/Load Flow“. The load flow uses only nodes and branches data and rewrite the input file with nodes data.

- „On line“ action:

- Some action is possible to carry out during the simulation by hotkeys F1 - F9..

- Press the selected hotkey  F.

- Write the  unit, node or branch name to the text box and blanks so that the first character of the name would on quit left.

- Use the key ®¬(with Num Lock offf) to move between box

- Use the space key to change state ON/OF (Zap/Vyp)

- Set the parameters according the table on the next page.

- Press the Enter to continue calculation.

- Graphic setup:

- Choose the menu „Modify/Graphics“ to open Graphic.

- Write 1/2/4 to the text box Number of Graphs and the range for Y Axis for the each chart

- Check the Complex Plane for complex mode

- Press the button Add Variables to open dialogue for adding another variable

- Select the object from Object list and the variable type in the frame Object Type

- Select the variable from the Variable Description list  (or click on the button in block scheme)

- Press the Add button to join the variable to the list of the variables (up to 7 is possible) and Escape.

- Write the text to the Label box.

- Check the  Step Changes box to see step changes during network commutations

- Press the OK to rewrite the original input file (option No) or create new input file variation (option No).

- You can use menu „Cases/Save As“ to creation of new case if you have variations.

- The following table gives overview of possible scenario, automatics/logics and hotkey actions:

| Code | Description | Name of | 1st param. | 2nd param. | Hotkeya |
| --- | --- | --- | --- | --- | --- |
| 'AUTO' | Automaton activation (I=1)/ blocking (I=0) | Automaton | I |  |  |
| 'STAB' | Stabiliser activation (I=1)/ blocking (I=0) | Stabiliser | I |  |  |
| 'LOGC' | Logic activation (I=1)/ blocking (I=0) | Logic | I |  |  |
| 'BRAN' | Branch state–three/one phase OFF (I=0/1), ON (3) switching off in from/to node (-4/4) | Branch | I |  | F1 |
| 'TRAN' | change of transformer ratio | Branch | abs{dPt}[pu] |  | F2 |
| 'STRC' | change of turbine control mode | Unit | I |  | F8 |
| 'EXCT' | change of excitation control set value | Unit | dU/dQ[pu] |  | F6 |
| 'TURB' | change of prime mover control set value | Unit | dN/dW[pu] |  | F7 |
| 'LOAD' | step change of load dPload and dQload | Node | dPload[%]* | dQload | F4 |
| 'UNIT' | Unit ON (I=1) OFF (I=0) | Unit | I |  | F5 |
| 'LFCS' | change the area LFC setup value | Area | Plan[MW] |  |  |
| 'LFCB' | secondary P/f control OFF(I=0) ON(I=1) | Unit | I |  |  |
| 'PUMP' | switching to pumping mode I=1 (for hydro turbine) | Unit | I |  |  |
| 'VALV' | activation (I=1)/ blocking (I=0) of fast valving | Unit | I |  |  |
| 'FREQ' | change of frequency set value df for prime mover control | Unit | df[pu] |  |  |
| 'SYNC' | synchronisation | Unit | 1 |  |  |
| 'ANAL' | carry out of network analysis | Area |  |  |  |
| 'FOUL' | short circuit located in Loc from beginning (from node) | Branch | Loc[%] | Xshunt[W] |  |
| 'CLER' | short circuit clearing: I=3;SC+1phase I=1;SC+3phases I=0 beginning (from node) I= -2; end (to node) I= 2 | Branch | I |  |  |
| 'EXCH' | additional harmonic signal into excitation control | Unit | amplitude[pu] | w[rad/s] |  |
| 'TURH' | additional harmonic signal into prime mover control | Unit | amplitude[pu] | w[rad/s] |  |
| 'EXCS' | additional step signal into excitation control | Unit | step[pu] |  |  |
| 'TURS' | additional step signal into prime mover control | Unit | step[pu] |  |  |
| 'MARK' | mark the number on the ith trajectory in graphic | ‘i’ | number [1-9] |  |  |
| 'FICT' | set the logical state on value I | Automaton | I [1/0] |  |  |
| 'RAMP' | ramp change of load dS rate | Node | dS [%] | rate[%/min]* |  |
| 'ISLN' | switch to the island control mode I=1 | Unit | I |  |  |
| 'PAR#'** | change the model parameters | Unit | i *** |  |  |

* percentile increasing from the initial value

**  # is sequential number of section in the parameter catalogue

***  i is sequential number of parameter set ,1 is for the 1st set in the global catalogue ( „default“ parameters) following the sets from local catalogue and then rest of the global catalogue

User Output Files Set-up

The MODES stores the time courses of selected units and network variables into ASCII files (located in VYST subdirectory). These files can serve for post-processing by external  spreadsheets or by MODMAN tool.

- Select the the UserFile from menu Modify to open the dialogue

- Press the Add User File button, select the file type eg. Variables from Display (then the file content corresponds to the variables in the cart in the graphics).

- Check the Use the Generic Name to force files names specified in the Generic Name text box. If it is empty the MODES uses name of the case. Uncheck the Use the Generic Name to specify own file name with path.

- Press the Add button to update files list (up to 12 files) and Escape button.

- Close the dialogue by OK or Cancel.

- View of the User Output Files:

- The MODMEN makes possible to see output files in text (standard option) and graphical mode (checked Results/As Graph when the calculation is finished.

- Choose the menu Results and select the user file type (the MODMAN shows available files if you use generic names)

- Select the file by Select button in File Selector dialogue.

- The chart is showed in graphical mode (checked As Graph) . Press the Copy to the Clipboard button to copy the chart in the WMF format. Then the it is possible to use paste or paste special to import chart to the documents.

- The text editor is used in text mode  (unchecked As Graph ). Standard text editor is NOTEPAD and user can replace it be own selection by menu Option.

- Reports

- The MODES writes the messages into report. This reports is accessed by menu Reports:

- Initialisation - information about input data reading.

- Events - information about events and action during simulation

- Step -information about integration step changing and CPU time.

- Init. Conditions -dynamic model initialisation and initial condition calculation

- Motor Conditions - initialisation of asynchronous motors model .

- Check reports about exceeding of limits for network variables (predefined by menu the menu „Modify/Check“)

- Exceeding of limits are signalised by pictograms in the graphic.:

- Analysis

- The MODES provides the analysis:

- The network state (initial, final and difference between them)

- The primary frequency control

- The secondary P/f control (load frequency control).

- The requirements on the analysis should be set before simulation by the menu „Modify/Analysis“. After the simulation are results available by the menu „Analysis“.

### Projects samples

- The following ready projects are copied during the package installation:

- SHORT - short term dynamics after short circuit in the network

- MIDDLE - mid term dynamics solving the frequency stability after unit outages till 30 second

- LONG - long term dynamics solving the frequency and voltage after unit outages till 30 minutes

- MOTOR - motors start -up and back-up in home consumption

- ISLAND - island operation and its re-synchronisation

- DIESEL - back-up sources modelling

- SHORT-Short term dynamics

- This project contains the following cases:

- UST_STAV 	initial steady state without any action

- ZKRAT_3F	three phase short circuit cleared by scenario

- CCT_15	critical clearing time calculation

- CCT_5	critical clearing time calculation with automatic stability check

- ZKRATAUT	three phase short circuit cleared by automaton

- KOMPLEX	the appearance impedance is showed in the complex together with automaton characteristic.

The network scheme with the short circuit is as follows:

Sch. 1 Scheme of the short circuit

MIDDLE-Mid term dynamics

This project contains the following cases:

- ZAKLADNI 	basic case of unit outages shows the primary frequency control operation

- MODIFN12	change the primary reserve on unit N12

- FREKVODL	frequency load shedding after unit outage.

- LONG-Long term dynamics

- This project contains the following cases:

- ZAKLADNI 	corresponds  basic case from the preceding project with calculation till 7 minutes without LFC control

- ACTIVELFC	activation of load frequency control

- NAPETKOL	voltage collapse simulation /see the following figure:

Sch. 2 Scheme of voltage collapse

The voltage collapse is initiated by the following events:

- ramp change of load in the N204 a N206 nodes 80% by rate  20%/min.

- unit M4 outage at  t=3min.

- double line  V451 a V452 outage at t=5 min.

- line V2_3 outage at t=6 min

MOTOR-Asynchronous motor operation

This project contains the following cases:

- ZASKOK  	the home consumption back-up by undervoltage relay  (see the following scheme).

- SYNCHRO 	the home consumption back-up with the synchrocheck relay

- START	asynchronous motor start-up

- Sch. 3 Scheme of the home consumption back-up by undervoltage relay

- ISLAND-Island operation

- This project contains the following cases:

- ISLAND	transit to the island (see the following scheme) in primary control mode

- PRIMREG	like ISLAND but the steam turbine on unit BLOKU3 are replaced by hydro one

- OTACREG  	transit to the island  in speed  control mode

- OSTRREG 	transit to the island  in island  control mode

- OTACREGP	transit to the island  in speed  control mode with power surplus

- OSTRREGP 	transit to the island  in island  control mode with power surplus

- STARTREZ 	fast unit start-ups

- FAZOVANI 	re-synchronisation of the island by synchrocheck

- Sch. 4 Scheme of transit to the island

- DIESEL-Island operation with the diesel-generator

- This project contains the following cases:

- USTSTAV	initial steady state of network feeded by diesel-generator

- ZKRATKOM   short circuit on diesel-generator terminal

- SKOKQZAT 	step change of reactive load

- STEPTEST	step change of active load

- ROZBEHAS	asynchronous motor start-up from the diesel-generator

- ROZBBRZD 	asynchronous motor start-up with the diesel-generator braking

- ROZBZMEN	asynchronous motor start-up with changing of the diesel-generator parameters.

Is directory with the same name like project containing two subdirectiries VST (with input data files) and VYST (with output files)

Is directory selected during installation, it contains subdirectories with input data and executable files

four types of input files are recognised by icons - load flow files, tables, databases and forms